# ROS 2 Humble Publisher Setup Guide (with Python Code)

This document explains step-by-step how to create and run a ROS 2 publisher that sends commands to a Teensy 4.1 subscriber via micro-ROS.

## Target Environment

- OS: Ubuntu 22.04 (Native)

- ROS Version: ROS 2 Humble

- Language: Python (rclpy)

## 1. Create a ROS 2 Workspace

mkdir -p ~/science_ws/src
cd ~/science_ws

## 2. Create a Python Publisher Package

cd src
ros2 pkg create science_keyboard_control --build-type ament_python

This creates the package:

science_keyboard_control/

## 3. Edit package.xml

Ensure the following dependencies are present:

<buildtool_depend>ament_python</buildtool_depend>
<exec_depend>rclpy</exec_depend>
<exec_depend>std_msgs</exec_depend>

## 4. Edit setup.py

Add the console entry point:

entry_points={
    'console_scripts': [
        'science_keyboard = science_keyboard_control.keyboard_node:main',
    ],
},

## 5. Python Publisher Node

Create the file:

science_keyboard_control/science_keyboard_control/keyboard_node.py

### keyboard_node.py

import rclpy
from rclpy.node import Node
from std_msgs.msg import Int32MultiArray
import sys
import termios
import tty

class KeyboardPublisher(Node):

    def __init__(self):
        super().__init__('science_keyboard_publisher')
        self.publisher_ = self.create_publisher(
            Int32MultiArray,
            '/science_control',
            10
        )
        self.get_logger().info('Science keyboard publisher started')

    def get_key(self):
        fd = sys.stdin.fileno()
        old_settings = termios.tcgetattr(fd)
        try:
            tty.setraw(fd)
            key = sys.stdin.read(1)
        finally:
            termios.tcsetattr(fd, termios.TCSADRAIN, old_settings)
        return key

    def run(self):
        msg = Int32MultiArray()

        while rclpy.ok():
            msg.data = [0, 0, 0, 0]
            key = self.get_key()

            if key == '\x1b':  # arrow keys
                k2 = self.get_key()
                k3 = self.get_key()

                if k3 == 'A':      # UP
                    msg.data[0] = 1
                elif k3 == 'B':    # DOWN
                    msg.data[0] = -1
                elif k3 == 'C':    # RIGHT
                    msg.data[1] = 1
                elif k3 == 'D':    # LEFT
                    msg.data[1] = -1

            elif key == 'b':
                msg.data[2] = 1

            elif key == 's':
                msg.data[3] = 1

            self.publisher_.publish(msg)
            self.get_logger().info(f'Published: {msg.data}')


def main():
    rclpy.init()
    node = KeyboardPublisher()
    node.run()
    node.destroy_node()
    rclpy.shutdown()


if __name__ == '__main__':
    main()

## 6. Build the Workspace

cd ~/science_ws
colcon build
source install/setup.bash

## 7. Run the Publisher

ros2 run science_keyboard_control science_keyboard

This publishes messages to:

/science_control

## 8. Verify Publishing

ros2 topic echo /science_control

You should see output like:

data: [1, 0, 0, 0]

## 9. Next Step

Start the micro-ROS agent and connect the Teensy 4.1 subscriber to receive these messages.

End of document.