# ROS 2 Humble – Science Subscriber Setup Guide

Workspace Name: science_ws

This document explains how to set up a ROS 2 Humble Python subscriber that listens to the topic science_sensor_data published by a Teensy 4.1 using micro-ROS.

## 1. Prerequisites

- Ubuntu 22.04

- ROS 2 Humble installed

- micro-ROS Agent installed

- Teensy 4.1 publishing data

Source ROS 2:

source /opt/ros/humble/setup.bash

## 2. Create the Workspace

mkdir -p ~/science_ws/src
cd ~/science_ws

## 3. Create the Python Subscriber Package

cd ~/science_ws/src
ros2 pkg create science_sub_py --build-type ament_python --dependencies rclpy std_msgs

This creates the following structure:

science_ws/
└── src/
    └── science_sub_py/
        ├── science_sub_py/
        │   ├── __init__.py
        │   └── science_subscriber.py
        ├── package.xml
        ├── setup.cfg
        └── setup.py

## 4. Subscriber Code

Edit the file:

science_ws/src/science_sub_py/science_sub_py/science_subscriber.py

Paste the Python subscriber code that subscribes to science_sensor_data using Float32MultiArray.

The subscriber: - Decodes all 14 values - Prints sensor information - Validates array length

## 5. Register the Entry Point

Edit setup.py and ensure the following entry exists:

entry_points={
    'console_scripts': [
        'science_subscriber = science_sub_py.science_subscriber:main',
    ],
}

This allows ROS 2 to run the node using ros2 run.

## 6. Build the Workspace

cd ~/science_ws
colcon build

After build completes, source the workspace:

source install/setup.bash

(Optional) Add this line to your ~/.bashrc for automatic sourcing:

echo "source ~/science_ws/install/setup.bash" >> ~/.bashrc

## 7. Run the Subscriber

Ensure the micro-ROS agent is running:

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/ttyACM0

In a new terminal:

source ~/science_ws/install/setup.bash
ros2 run science_sub_py science_subscriber

## 8. Verify the Setup

Check available topics:

ros2 topic list

Check topic details:

ros2 topic info /science_sensor_data

Echo raw data:

ros2 topic echo /science_sensor_data

## 9. Common Issues & Fixes

### Node not found

- Ensure colcon build completed successfully

- Source install/setup.bash

### No data received

- Verify micro-ROS agent is running

- Check Teensy is detected using /dev/ttyACM0

- Confirm topic name matches exactly

### Permission denied on serial port

sudo usermod -a -G dialout $USER
reboot

## 10. Next Steps

- Forward data to REST or MQTT

- Convert array to custom ROS message

- Add logging and timestamps

- Integrate cloud pipelines

End of Document