# usbipd → WSL → micro-ROS (Serial) Command Cheat Sheet

This file lists all commands in correct order, starting from discovering the USB device on Windows up to running the micro-ROS agent in WSL using serial transport.

## 🪟 Windows (PowerShell — Run as Administrator)

### 1️⃣ List connected USB devices

usbipd list

Example output:

BUSID  VID:PID    DEVICE
1-2    16C0:0483  Teensy USB Serial

➡️ Note the BUSID (example: 1-2)

### 2️⃣ Bind the USB device (one-time)

usbipd bind --busid 1-2

### 3️⃣ Attach the USB device to WSL

usbipd attach --busid 1-2 --wsl

(If you have multiple distros)

usbipd attach --busid 1-2 --distribution Ubuntu-22.04

## 🐧 WSL (Ubuntu)

### 4️⃣ Verify the serial device appears

ls -l /dev/ttyACM*

Expected:

/dev/ttyACM0

### 5️⃣ (Optional) Check kernel messages

dmesg | tail

### 6️⃣ Fix serial permissions (one-time)

sudo usermod -aG dialout $USER
newgrp dialout

### 7️⃣ Run micro-ROS agent in serial mode

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/ttyACM0 -v6

## 🔁 Reconnect workflow (when unplugging or reflashing)

### Windows

usbipd detach --busid 1-2
usbipd attach --busid 1-2 --wsl

### WSL

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/ttyACM0 -v6

## ✅ Expected successful output

[INFO] Serial agent started
[INFO] [XRCE] Session established
[INFO] Node created

## ⚠️ Notes

- Do NOT flash firmware while USB is attached to WSL

- BUSID may change after unplug/replug

- Always start the agent after USB is attached

- Use rmw_uros_ping_agent() in firmware to avoid timing issues

✅ End of command sequence